# Andre Miranda

CS-350 Emerging Sys Arch & Tech

Professor Sina Asadi

June 16, 2026

7-1 Final Project Report

The thermostat prototype developed for this project demonstrates the essential embedded functionality required for a modern smart thermostat system. The device integrates temperature sensing, LED indicators, user input buttons, an LCD display, and UART communication to simulate cloud reporting. These features collectively satisfy SysTec’s business requirement for a functional low level thermostat prototype before advancing to cloud integration (SysTec, 2024). The system operates using a finite state machine that transitions between OFF, HEAT, COOL, and back to OFF modes based on user input. A green button cycles through these states, while red and blue buttons adjust the temperature set point. The LCD continuously displays the date and time on the first line and alternates between the current temperature and the system’s operational state on the second line. This alternating display ensures that the user receives both environmental information and system status in a clear and predictable manner, consistent with the specifications outlined in the project guidelines (Lab Guide, 2024).

The thermostat supports all required peripherals through the Raspberry Pi’s built-in interfaces. The AHT20 temperature sensor communicates over I2C, allowing the system to read accurate temperature and humidity values. These readings are essential for determining whether heating or cooling should be active. The Raspberry Pi’s GPIO pins control two LEDs that indicate heating and cooling status. When the system is in heating mode and the temperature is below the set point, the red LED pulses; when the temperature reaches or exceeds the set point, the LED becomes solid. Similarly, in cooling mode, the blue LED pulses when cooling is required and becomes solid when the temperature falls to or below the set point. The three buttons used for state cycling and temperature adjustment are managed through GPIO interrupts using the GPIOZero library. This ensures responsive input handling without blocking the main program loop, fulfilling the requirement for interrupt-driven button processing (SysTec, 2024). The LCD display, connected through GPIO, provides real-time feedback to the user by showing the date, time, temperature, humidity, and system state. Finally, the UART peripheral is configured to output a comma-delimited string containing the thermostat’s state, current temperature, and set point every 30 seconds, simulating data transmission to a cloud server.

As SysTec prepares for the next phase of development, the thermostat must eventually connect to the cloud via Wi-Fi. To determine the best hardware architecture for the production version, three platforms were evaluated: Raspberry Pi, Microchip microcontrollers, and Freescale/NXP processors. The Raspberry Pi 4B includes built-in Wi-Fi, substantial RAM, and flexible software support, making it ideal for rapid prototyping and early cloud integration testing. Its Linux environment and Python support allow for fast development cycles and easy integration with cloud APIs. However, the Raspberry Pi is more expensive and consumes more power than microcontroller-based solutions, making it less suitable for mass-produced thermostats (SysTec, 2024). Microchip’s PIC32 and SAM series microcontrollers offer low-power operation, integrated Wi-Fi modules such as the WINC1500, and sufficient Flash and RAM to support embedded IoT applications. These devices also support I2C, UART, and GPIO, meeting all peripheral requirements of the thermostat. Their low cost and reliability make them strong candidates for commercial deployment (Microchip, 2023). Freescale/NXP architectures, such as the Kinetis and i.MX RT series, also provide robust peripheral support, scalable memory configurations, and integrated wireless options. These processors are widely used in commercial IoT devices and offer robust performance with efficient power consumption (NXP, 2023).

Based on the analysis of these architectures, the Raspberry Pi remains the best choice for prototyping due to its ease of development and built-in Wi-Fi capabilities. However, for a production ready thermostat, Microchip microcontrollers offer the most balanced solution in terms of cost, power efficiency, and integrated wireless support. Their architecture aligns well with SysTec’s business requirements for cloud connectivity, peripheral support, and sufficient memory resources (Microchip, 2023). This makes Microchip the recommended platform for the commercial version of the thermostat.

References

Lab Guide. (2024). CS 350 Final Project Thermostat Lab Guide. Southern New Hampshire

University.

Microchip. (2023). Microchip IoT and wireless solutions overview. Microchip Technology Inc.

NXP. (2023). Kinetis and i.MX RT product families. NXP Semiconductors.

SysTec. (2024). CS 350 Final Project Guidelines and Rubric. Southern New Hampshire

University.