# Andre Miranda

CS-350 Emerging Sys Arch & Tech

Professor Sina Asadi

June 04, 2026

Milestone Three: Input With Buttons Lab

1. Why does the LED-blinking loop need to run in a separate thread?

The LED-blinking loop must run in a separate thread because Morse code timing requires precise, blocking delays (sleep() calls). Each dot, dash, and pause introduces waits ranging from 250ms to 3000ms. If this loop ran on the main thread, the program would become unresponsive during these waits. By placing the transmission loop in its own thread, the system can continue detecting button presses immediately, update the 16×2 LCD display without delay, maintain a responsive state machine, and avoid blocking the main control loop. This separation ensures that real-time input (button events) and time-sensitive output (LED blinking) operate smoothly together, which is essential in embedded systems.

2. What is the purpose of returning to the off state after each completed state action?

Returning to the off state after every dot, dash, or pause ensures the state machine always begins the next action from a known, stable baseline. This is important because it prevents LEDs from being left unintentionally on, guarantees consistent timing between Morse elements, simplifies transitions since every next state starts from off, and avoids overlapping or conflicting LED states.  Returning to the off state emphasizes predictable transitions and clean state boundaries because the off-state acts as a reset point, ensuring the Morse code output remains accurate and readable.

3. How could you integrate serial communications to facilitate changing the messages available to the program?

Serial communication could be added as a remote command interface that allows an external system, including PC, microcontroller, or higher-level controller, to send new messages to the Raspberry Pi. The Pi could listen on /dev/ttyS0 for commands such as:

- SET SOS

- SET OK

- SET HELLO WORLD

- NEXT (cycle to next message)

The program would then:

- Read incoming bytes from the UART

- Decode them into strings

- Validate the command

- Update activeMessage or the message list accordingly

This would allow dynamic message updates without touching the hardware or restarting the program. This mirrors the UART behavior demonstrated in Milestone Two, where data is encoded, transmitted, decoded, and acted upon.

4. How could you use the 16x2 display to provide debugging information to the user when they don’t have access to the application console?

The 16×2 LCD can act as a real-time status and debugging interface, especially when the user cannot see the terminal output by maybe outputting information like:

- The current message being transmitted

- The next message queued after a button press

- The current Morse element (e.g., “DOT”, “DASH”, “PAUSE”)

- State machine status (e.g., “STATE: dotDashPause”)

- Error messages if hardware fails to initialize

- Confirmation that the button was pressed

For example, the display might show:

Sending:

SOS

or during debugging:

BUTTON PRESSED

Next: OK

This allows the user to verify system behavior directly from the device, which is essential in embedded environments where console access is limited or unavailable.